# ПРИЛОЖЕНИЕ Б

# СОДЕРЖАНИЕ

Программный код ключевых фрагментов. Полные исходники: mur_ide/resources/examples/waypoint_mission/

waypoint_mission.py — подключение к симулятору и основной цикл 10 Гц.

waypoints.py — переход к следующей путевой точке (гистерезис).

navigation.py — расстояние, целевой курс, ошибка курса.

control.py — П-законы для моторов симулятора.

logger_module.py — заголовок CSV и запись строки.

odometry.py — оценка координат при отсутствии X/Y в API.

1. waypoint_mission.py — основной цикл

import pymurapi as mur

from waypoints import WaypointManager, load_waypoints_from_csv, default_waypoints

from navigation import calculate_target_yaw_near_aware, distance_to_waypoint

from control import sim_motor_commands, apply_motor_commands

from logger_module import Logger

from odometry import step_dead_reckoning

CAPTURE_RADIUS = 0.10

REACH_SLACK_M = 0.05

APPROACH_RING_M = 0.06

UPDATE_HZ = 10.0

LOG_INTERVAL = 1.0

def main() -> None:

auv = mur.mur_init()

if auv is None:

return

manager = WaypointManager(

waypoints=load_waypoints_from_csv("waypoints.csv"),

capture_radius=CAPTURE_RADIUS,

reach_slack_m=REACH_SLACK_M,

approach_ring_m=APPROACH_RING_M,

cycles=2,

)

logger = Logger()

est_x, est_y = 0.0, 0.0

last_m0, last_m1 = 0, 0

while not manager.is_finished():

x, y = est_x, est_y  # или get_x/get_y из API, если доступны

z = float(auv.get_depth())

current_yaw = float(auv.get_yaw())

wp = manager.current()

distance = distance_to_waypoint((x, y), wp)

target_yaw = calculate_target_yaw_near_aware(

x, y, wp[0], wp[1], current_yaw, degenerate_tol_m=0.08

)

motors = sim_motor_commands(current_yaw, target_yaw, z, wp[2])

apply_motor_commands(auv, motors)

last_m0, last_m1 = motors[0], motors[1]

est_x, est_y = step_dead_reckoning(

est_x, est_y, current_yaw, last_m0, last_m1, dt=0.1

)

manager.advance_if_reached(distance)

logger.log(x, y, z, current_yaw, target_yaw, distance, motors, manager._current_index)

time.sleep(1.0 / UPDATE_HZ)

2. waypoints.py — схватывание точки

def advance_if_reached(self, distance_to_current: float) -> None:

r_in = self.capture_radius + self.reach_slack_m

r_out = self.approach_ring_m

if distance_to_current > r_out:

self._was_outside_capture_for_current = True

if distance_to_current > r_in:

return

if not self._was_outside_capture_for_current:

return

self._was_outside_capture_for_current = False

self._current_index += 1

if self._current_index >= len(self.waypoints):

self._current_index = 0

self._completed_cycles += 1

3. navigation.py — расчёты навигации

def calculate_distance(x1, y1, x2, y2) -> float:

dx, dy = x2 - x1, y2 - y1

return math.sqrt(dx * dx + dy * dy)

def calculate_target_yaw(x_cur, y_cur, x_tgt, y_tgt) -> float:

return math.degrees(math.atan2(y_tgt - y_cur, x_tgt - x_cur))

def calculate_target_yaw_near_aware(x_cur, y_cur, x_tgt, y_tgt, current_yaw_deg, degenerate_tol_m=0.08):

dx, dy = x_tgt - x_cur, y_tgt - y_cur

if dx * dx + dy * dy < degenerate_tol_m ** 2:

return float(current_yaw_deg)

return calculate_target_yaw(x_cur, y_cur, x_tgt, y_tgt)

def calculate_yaw_error(target_yaw, current_yaw) -> float:

return math.degrees(math.atan2(

math.sin(math.radians(target_yaw - current_yaw)),

math.cos(math.radians(target_yaw - current_yaw)),

))

4. control.py — П-регуляторы курса и глубины

SIM_BASE_FORWARD = 40

SIM_YAW_RES_GAIN = -0.8

SIM_DEPTH_GAIN = 80.0

def sim_motor_commands(current_yaw, target_yaw_deg, depth_measured, depth_target, base_forward=40):

er = calculate_yaw_error(target_yaw_deg, current_yaw)

res = max(-28, min(28, SIM_YAW_RES_GAIN * er))

m0 = int(max(-100, min(100, base_forward - res)))

m1 = int(max(-100, min(100, base_forward + res)))

depth_cmd = SIM_DEPTH_GAIN * (depth_measured - depth_target)

m2 = m3 = int(max(-100, min(100, depth_cmd)))

return (m0, m1, m2, m3)

5. logger_module.py — запись CSV

LOG_HEADER = [

"Время", "X", "Y", "Z", "Курс", "Целевой_курс", "Расстояние",

"Мощность_0", "Мощность_1", "Мощность_2", "Мощность_3", "Номер_точки",

]

def log(self, x, y, z, yaw, target_yaw, distance, motors, waypoint_index):

row = [time.time() - self._start_time, x, y, z, yaw, target_yaw, distance, *motors, waypoint_index]

self._file.writerow(row)

6. odometry.py — оценка положения

ODOMETRY_SPEED_SCALE = 0.065

def step_dead_reckoning(x, y, yaw_deg, motor0, motor1, dt):

yaw_rad = math.radians(yaw_deg)

v = (0.5 * (motor0 + motor1) / 100.0) * ODOMETRY_SPEED_SCALE

x += v * math.sin(yaw_rad) * dt

y += v * math.cos(yaw_rad) * dt

return x, y

Модуль visualize.py (построение графика OXY по CSV) приведён в репозитории целиком; в пояснительной записке описан в п. 3.7.